# UN/SCETDG/30/INF.6

COMMITTEE OF EXPERTS ON THE TRANSPORT OF

DANGEROUS GOODS AND ON THE GLOBALLY

HARMONIZED SYSTEM OF CLASSIFICATION

AND LABELLING OF CHEMICALS

Sub-Committee of Experts on the

Transport of Dangerous Goods

Thirtieth session

Geneva, 4th-12th December 2006

Item 5 of the provisional agenda

GUIDING PRINCIPLES FOR THE MODEL REGULATIONS

Corrigendum to ST/SG/AC.10/C.3/2006/86

Transmitted by the expert from the United Kingdom

In the Table of Limited Quantities on page 15, there is an error in the entry for Class 2 substances.  The complete table is reproduced here with this amended.

| CLASS | PG | Quantity |
| --- | --- | --- |
| 1 |  | Not permitted |
| 2.1 |  | Not permitted |
| 2.2 (without subsidiary risk) |  | 120ml/30kg 1L/30kg - for aerosols not containing toxic substances |
| 2.3 |  | Not permitted |
| 3 | I | Not permitted |
| 3 | II | 1 L/30kg (see Note 1); desensitised explosives not permitted |
| 3 | III | 5 L/30kg |
| 4.1 | II | 1 kg/30kg; desensitised explosives and self-reactive substances not permitted |
| 4.1 | III | 5 kg/30kg; desensitised explosives and self-reactive substances not permitted |
| 4.3 | II | 500 g/30kg |
| 4.3 | III | 1 kg/30kg |
| 5.1 | II | 500 g/30kg |
| 5.1 | III | 5L - 5 kg /30kg |
| 5.2 Liquid | II | (For types D, E or F 125 ml/30kg; for types B or C 25ml/30kg, except no temperature controlled substances |
| 5.2 Solid | II | (For types D, E or F 500g/30kg; for types B or C 100g/30kg |
| 6.1 | I | Not permitted |
| 6.1 | II | 100 ml - 500g/30kg |
| 6.1 | III | 5 L – 5kg/30kg |
| 8 | I | Not permitted |
| 8 | II | 1L - 1 kg/30kg |
| 8 | III | 5 L - 5kg/30kg |
| 9 | II | 1L - 1kg/30kg (see Note 2) |
| 9 | III | 5 L - 5kg/30kg |

______________