|  | United Nations | ECE/TRANS/WP.29/2011/130 |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 29 July 2011 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

155th session

Geneva, 15–18 November 2011

Item 4.11.3 of the provisional agenda

1958 Agreement – Consideration of draft corrigenda

to existing Regulations submitted by GRE

Proposal for Corrigendum 6 to the 03 series of amendments to Regulation No. 19 (Front fog lamps)

Submitted by the Working Party on Lighting and Light-Signalling*

The text reproduced below was adopted by the Working Party on Lighting and Light-Signalling (GRE) at its sixty-fifth session to correct a conflict between some specifications for the horizontal position and the diagram in Annex 4. It is based on informal document GRE-65-27, as reproduced in Annex IX to the report (ECE/TRANS/WP.29/GRE/65, para. 39). It is submitted to the World Forum for Harmonization of Vehicle Regulations (WP.29) and to the Administrative Committee (AC.1) for consideration.

Paragraph 6.4.3., amend to read:

"6.4.3.	Photometric requirements

When so adjusted, the front fog lamp shall meet the photometric requirements in the table below (refer also to Annex 4, paragraph 2.2. of this Regulation):

| Designated lines or zones | Vertical position */ above h + below h - | Horizontal position */ left of v: - right of v: + | Luminous intensity (in cd) | To comply |
| --- | --- | --- | --- | --- |
| Point 1, 2 ** | + 60° | ± 45° | 60 max | All points |
| Point 3, 4 ** | + 40° | ± 30° |  |  |
| Point 5, 6 ** | + 30° | ± 60° |  |  |
| Point 7, 10 ** | + 20° | ± 40° |  |  |
| Point 8, 9 ** | + 20° | ± 15° |  |  |
| Line 1 ** | + 8° | - 26° to + 26° | 90 max | All line |
| Line 2 ** | + 4° | - 26° to + 26° | 105 max | All line |
| Line 3 | + 2° | - 26° to + 26° | 170 max | All line |
| Line 4 | + 1° | - 26° to + 26° | 250 max | All line |
| Line 5 | 0° | - 10° to + 10° | 340 max | All line |
| Line 6 *** | - 2.5° | - 10° to + 10° | 2,000 min | All line |
| Line 7 *** | - 6.0° | - 10° to + 10° | < 50 per cent of max. on line 6 | All line |
| Line 8L and R *** | -1.5° to - 3.5° | - 22° and + 22° | 800 min | One or more points |
| Line 9L and R *** | -1.5° to - 4.5° | - 35° and + 35° | 320 min | One or more points |
| Zone D *** | - 1.5° to - 3.5° | - 10° to + 10° | 8,400 max | Whole zone |
| * The co-ordinates are specified in degrees for an angular web with a vertical polar axis. ** See paragraph 6.4.3.4. *** See paragraph 6.4.3.2. |  |  |  |  |

"

Paragraph 6.4.3.2., amend to read:

"6.4.3.2. 	At the request of the applicant, two front fog lamps constituting a matched pair corresponding to paragraph 4.2.2.5. may be tested separately. In this case the specified requirements for lines 6, 7, 8, 9 and the Zone D in the table in paragraph 6.4.3. apply to half the sum of readings of the right-hand and left-hand side front fog lamp. However each of the two front fog lamps shall meet at least 50 per cent of the minimum value required for line 6. Additionally, each of the two front fog lamps that constitute the matched pair corresponding to paragraph 4.2.2.5. are only required to meet the requirements of line 6 and line 7 from 5° inwards to 10° outwards."

*	*	In accordance with the programme of work of the Inland Transport Committee for 2010–2014 (ECE/TRANS/208/, para. 106 and ECE/TRANS/2010/8, programme activity 02.4), the World Forum will develop, harmonize and update Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.